### Assigning a MAC Address:-

A MAC address is a unique identifier assigned to a network interface card (NIC) at the hardware level. You cannot change the MAC address of a device physically, but you can change it software-wise on most devices. Here's how you can do it on different systems.

### 1. Change MAC Address on Windows

#### Steps to Change MAC Address:

- Open Device Manager:

  - Press Windows + X and select Device Manager.

- Find the Network Adapter:

  - Under Network adapters, right-click on your network adapter (either Ethernet or Wi-Fi) and select Properties.

- Go to the Advanced Tab:

  - In the Properties window, navigate to the Advanced tab.

- Find the "Network Address" or "Locally Administered Address":

  - Scroll down in the list until you see Network Address or Locally Administered Address.

- Change the MAC Address:

  - Enter a new MAC address in the Value field. It should be 12 characters long, using hexadecimal digits (0-9, A-F), and format it as 00:11:22:33:44:55.

- Apply the Changes:

  - Click OK to save. Disable and re-enable your network adapter to apply the change.

### 2. Change MAC Address on macOS :-

#### Steps to Change MAC Address:

- Open Terminal:

  - Navigate to Applications > Utilities > Terminal.

- Find the Network Interface:

  - Type the following command to see the network interfaces:

bash

Copy

ifconfig

  - Identify your network interface (commonly en0 for Wi-Fi).

- Disable the Interface:

  - To disable the interface, use the following command:

bash

Copy

sudo ifconfig en0 down

- Change the MAC Address:

  - Change the MAC address with this command:

bash

Copy

sudo ifconfig en0 ether xx:xx:xx:xx:xx:xx

  - Replace xx:xx:xx:xx:xx:xx with your desired MAC address.

- Re-enable the Interface:

  - To enable the interface again, use:

bash

Copy

sudo ifconfig en0 up

### 3. Change MAC Address on Linux :-

#### Steps to Change MAC Address:

- Open Terminal:

  - Open the terminal on your Linux machine.

- Find the Network Interface:

  - Type the following command to list all network interfaces:

bash

Copy

ifconfig

  - Find your network interface (e.g., eth0 or wlan0).

- Disable the Interface:

  - Disable the network interface with:

bash

Copy

sudo ifconfig eth0 down

- Change the MAC Address:

  - Change the MAC address with:

bash

Copy

sudo ifconfig eth0 hw ether xx:xx:xx:xx:xx:xx

  - Replace xx:xx:xx:xx:xx:xx with the new MAC address.

- Re-enable the Interface:

  - Re-enable the network interface:

bash

Copy

sudo ifconfig eth0 up

### Assigning an IP Address:-

You can assign IP addresses in various contexts:

- On a local device (like your computer)

- On a router (for a network)

### 1. Assign Static IP Address on Windows

#### Steps to Assign a Static IP Address:-

- Open Network Settings:

  - Right-click on the Network icon in the taskbar and select Open Network & Internet Settings.

  - Click on Change adapter settings.

- Select Network Adapter:

  - Right-click on your network adapter (either Wi-Fi or Ethernet) and select Properties.

- Select IPv4:

  - In the Properties window, scroll down and select Internet Protocol Version 4 (TCP/IPv4), then click Properties.

- Assign Static IP:

  - In the IPv4 properties window, select Use the following IP address.

  - Enter your desired IP address, subnet mask (usually 255.255.255.0), and gateway (your router's IP address, typically 192.168.1.1).

Example:

  - IP Address: 192.168.1.100

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

- Save Settings:

  - Click OK to save the settings.

### 2. Assign Static IP Address on macOS:-

#### Steps to Assign a Static IP Address:

- Open Network Settings:

  - Go to System Preferences > Network.

- Select Network Interface:

  - Select your active network interface (Wi-Fi or Ethernet) from the left panel.

- Configure IPv4:

  - Click on the Advanced button, then go to the TCP/IP tab.

- Assign Static IP:

  - From the dropdown, choose Manually under Configure IPv4.

  - Enter your desired IP address, subnet mask (typically 255.255.255.0), and router (default gateway) IP address.

Example:

  - IP Address: 192.168.1.100

  - Subnet Mask: 255.255.255.0

  - Router: 192.168.1.1

- Save Settings:

  - Click OK, then click Apply to save the settings.

### 3. Assign Static IP Address on Linux :-

#### Steps to Assign a Static IP Address:

- Edit Network Configuration File:

  - Open a terminal and use a text editor like nano to open the network configuration file for your network interface. The file might be located in /etc/network/interfaces (on some distributions) or you may need to configure it via NetworkManager.

bash

Copy

sudo nano /etc/network/interfaces

Example configuration:-

bash

Copy

auto eth0

iface eth0 inet static

address 192.168.1.100

netmask 255.255.255.0

gateway 192.168.1.1

- Save and Apply:

  - Save the file and exit. Restart the network interface:

bash

Copy

sudo systemctl restart networking

### 4. Assign Static IP Address on a Router :-

#### Steps to Assign Static IP Address (DHCP Reservation) on a Router:

- Log into Router:

  - Open a web browser and enter the router's IP address (usually 192.168.1.1 or 192.168.0.1).

  - Enter the admin username and password.

- Find DHCP Settings:

  - Navigate to the DHCP Settings or LAN Setup section.

- Add DHCP Reservation:

  - Look for a section called DHCP Reservation or Static Lease.

  - Find your device by its MAC address and assign it a static IP.

Example:

  - Device: Laptop

  - MAC Address: 00:11:22:33:44:55

  - IP Address: 192.168.1.100

- Save and Reboot:

  - Save the settings, and the router will now always assign the same IP address to that device when it connects.

### Summary:-

- MAC Address: It’s a hardware address, which can be temporarily changed on most devices for privacy or testing purposes.

- IP Address: It’s used for network communication. Static IPs can be assigned manually in network settings on a device or through a router’s DHCP reservation.

By following these steps, you can configure both MAC and IP addresses to meet your specific needs. Let me know if you need more details on any of these steps!