### 1. Block WhatsApp Using Router/Firewall (For Entire Network):

This method blocks WhatsApp for any device connected to your Wi-Fi network, but it doesn't rely on ChatGPT or other software on the device itself. It uses your router's settings to block traffic to WhatsApp's servers.

- Steps:

  - Log in to your router's admin interface (usually through a web browser like 192.168.0.1 or 192.168.1.1).

  - Navigate to the Firewall or Security settings.

  - Add blocking rules to prevent access to WhatsApp's servers:

    - You can block domains such as:

      - web.whatsapp.com

      - whatsapp.com

      - *.whatsapp.net

    - Alternatively, block the IP ranges used by WhatsApp, though these can change over time.

  - Apply changes and restart your router.

### 2. Using Parental Control Software:

There are various apps and services you can use on devices to block apps like WhatsApp:

#### On Android:

- Using Digital Wellbeing or Parental Controls:

  - Go to Settings > Digital Wellbeing & Parental Controls.

  - Tap Parental Controls and set up restrictions to block WhatsApp.

  - Alternatively, you can use third-party apps like Qustodio or Family Link for more detailed control.

#### On iOS (iPhone/iPad):

- Using Screen Time:

  - Go to Settings > Screen Time.

  - Tap App Limits and add a time limit for WhatsApp.

  - You can also go to Content & Privacy Restrictions > Allowed Apps and disable WhatsApp entirely.

#### On Windows or macOS (Using Software):

- Using Third-Party Tools:

  - Windows: Tools like Cold Turkey or Kaspersky Safe Kids can block apps or websites like WhatsApp.

  - macOS: You can use Focus (Mac App Store) to block specific apps, or OpenDNS to block the WhatsApp website.

### 3. Blocking WhatsApp at the DNS Level (Using DNS Filtering):

If you prefer a network-wide solution to block WhatsApp at the DNS level, you can configure your device to use a service like OpenDNS or Cloudflare Family DNS.

- Steps:

  - Go to Settings > Network & Internet > Wi-Fi (on Windows) or Settings > Wi-Fi (on macOS/iOS).

  - Modify your DNS settings to use a custom DNS server:

    - OpenDNS: 208.67.222.222 and 208.67.220.220

    - Cloudflare Family DNS: 1.1.1.3 (blocks adult content, may help block WhatsApp)

  - Follow the setup guides on the OpenDNS or Cloudflare website to block specific websites.

### 4. Use Device-specific App Management (App-Specific Blocking):

If you want to block WhatsApp specifically on a device:

- On Android:

  - You can go to Settings > Apps > WhatsApp and force-stop the app or uninstall it.

- On iOS:

  - Go to Settings > General > iPhone Storage and find WhatsApp, then delete it.

- Block Notifications or Limit App Usage:

  - On both iOS and Android, you can limit WhatsApp usage via built-in features like Screen Time or Digital Wellbeing. You can also restrict notifications or completely block the app during certain hours.

### 5. Blocking with Third-Party Apps (for any platform):

There are a variety of apps that offer customizable blocking options for any platform. Here are a few you could consider:

- Freedom: Works on Windows, macOS, Android, and iOS. Allows you to block apps, websites, and other distractions.

- Net Nanny: Allows for full monitoring and blocking of specific apps and websites across different devices.

Would you like further guidance on any of these methods or specific step-by-step instructions for your device?